BUILDING BYE-LAWS

TEHSIL MUNICIPAL ADMINISTRATION, CHAKWAL

NOTIFICATION

No. TMA-CKL/1631	Dated 01-07-2003

In exercise of the powers conferred on me under Section 192 read with S.54(1)(e) and Clause 39 of 6th Schedule of the PUNJAB LOCAL GOVERNMENT Ordinance 2001, I, Ch. Muhammad Amir Khan Bhatti, Tehsil Nazim Chakwal, hereby enforce, with the approval of Tehsil Council Chakwal the under mentioned Building Bye-laws in super cession of the previous Building Bye-laws published in the Punjab Government Gazette Notification No.746/TPO/GB/MCC dated 13 November, 1995 for general information.

PART - 1

PRELIMINARY

1.	Short Title and Commencement.

i)	These bye-laws may be called Tehsil Municipal Administration Chakwal (Erection and Re-erection of Building Bye-laws 2003.

ii)	These Bye-laws shall extend to the whole Tehsil Chakwal except the areas exempted by the Tehsil Council, Chakwal.

iii)	These Bye-laws shall come into force with effect from 1st July, 2003.

2.	Regulation of Erection and Re-erection of Buildings.

i)	No person shall erect or re-erect a building or commence to erect or re-		erect a building on any land without prior sanction of Tehsil Municipal 		Administration Chakwal.

ii)	Tehsil Municipal Administration Chakwal may for reasons to be recorded 			in writing reject a site or building plan.

iii)	The Tehsil Municipal Administration Chakwal may sanction any erection 			or re-erection of a building either absolutely or subject to modifications as 			it deems necessary.

PART -II

GENERAL ADMINISTRATIVE MATTERS

3.	Interpretation of Terms.

Unless otherwise expressly stated, the following terms shall for the purpose of these Bye-laws have the meanings indicated in this part.  Where the terms are not defined they shall have their ordinarily accepted meaning of such as the context may apply.

i.	Administration - means Tehsil Municipal Administration Chakwal.

ii.	Approved Scheme - means a project approved by the competent authority for Urban Development.

iii.	Arcade - means a covered footpath for pedestrians in front of a shop in         commercial building.

iv.	Balcony - means a stage or platform projecting from the wall of a building surrounded with a railing or parapet wall.

v.	Base - means the underside of the course immediately above the footings applied to a wall or pillar.

vi.	Basement - means the lowest storey/storeys of a building, partly or wholly below the ground level.

vii.	Building Clerk - means the clerk of Planning & Coordination Office.

viii.	Building Inspector - means the building inspector of Planning & Coordination Office.

ix.	Building Works - means the site excavation, erection or re-erection of building or making additions/alterations.

- Building/House Line - means a line beyond which the outer face of any building except compound wall, may not project in the direction of any road or street existing or proposed.

xi.	Cardinal Points - means the directions of North, South, East and West.

xii.	Commercial Building - means a building having shops or show rooms, offices, hotel, private school, filling/servicing station etc. and may also have housing units/apartments.

xiii.	Competent Authority - means the Nazim, or authorized officer by the Nazim to approve plans and to control the building activity.

xiv.	Council - means Tehsil Municipal Council, Chakwal.

xv.	Damp Proof Course (DPC) - means a layer of material impervious to moisture.

xvi.	Draftsman - means draftsman of  P & C office.

xvii.	Floor Area Ratio - means the aggregate covered area of the building on a plot divided by the total area of the plot.

xviii.	Foundation - means a structure entirely below the level of the ground which carries and distributes the load from pillars, beams or walls on the ground.

xix.	Gallery - means an open or a covered walk-way or a long passage or an underground passage.

xx.	Government - means the Government of Punjab.

xxi.	Industrial Building - means a building designed for use as a factory or workshop and includes an office or other accommodation on the same site, which is intended for use in connection with the mining or minerals including brick or the earth or warehouse, depository or stores etc.

xxii.	Kacha Building - means a building having walls made of bricks/stones laid with mud, roof material grass, hay, straw, reed temporarily made for residence or animal shelter.

- Kanal - means a size of land equal to 20 marlas @ 272 sft. as in revenue papers and @ 248 sft. as otherwise described and generally applied in  Chakwal city.

xxiv.	Licensed Architect/Town Planner/Draftsman - means a person qualified as such enrolled on the list of approved architects/town planners/draftsmen maintained by the Tehsil Municipal Administration Chakwal.

xxv.	Market - means a group of shops assigned particularly for one or more specified trades.

xxvi.	Marla - means a spatial unit of 272 sft. as in revenue papers and 248 sft. as otherwise described and generally applied in Chakwal City.

xxvii.	Multi-storey Building - means any building having more than one storey.

xxviii.	Nazim - means Nazim of Tehsil Municipal Administration, Chakwal.

xxix.	Ordinary Repair - means painting white washing, plastering, paving and minor alterations to a building.

xxx.	Parapet - means a wall, whether plain, perforated or paneled, protecting the edge of a roof, balcony, verandah or terrace.

xxxi.	Pergola - means a structure with perforated roof consisting of cross bars in the form of re-enforced concrete, wood or steel etc. of which more than 50% of roof is open to the sky.

xxxii.	Place of Public Worship - means a building or institutes of building designed for a purpose associated with the performance of religious functions.

xxxiii.	Plinth - means the portion of the building between the level of the site and the level of the ground floor.

xxxiv.	Public Building - means a building designed for public use and may include a dispensary, post office, police station, tanga or transport stand, town hall, library, cultural centre and premises of social agencies such as hostels etc.

xxxv.	Residential Building - means a building exclusively designed for the use as a human habitation together with such other uses as are ordinarily ancillary to the main building and used in connection therewith.

xxxvi.	Semi Detached Building - means two buildings but on adjacent plots having a common partition walls.

xxxvii.	Septic Tank - means a tank in which sewage is collected and decomposed before its discharge into a public sewer or soakage pit.

xxxviii.	Soakage Pit - means a pit filled with aggregate, boulders or broken bricks and intended for the reception of waste water or effluent discharged from a septic tank.

xxxix.	Sq. Ft.  - means Square Feet.

xl	Structural Calculations - means detailed calculations showing sufficiency of the strength of every load bearing part of the proposed structures.

- Sun Shade - means an outside projection from a building over minimum height of 7 feet from the plinth level meant to provide protection from sun and rain.

- Structural Engineer -  means Civil Engineer having Msc Degree in Structural Engineering and registered with Pakistan Engineer Council as Structural Engineer

xliii.	Tehsil Municipal Administration (TMA) - means Tehsil Municipal Administration Chakwal.

xliv.	Tehsil Officer Planning & Coordination TO (P&C) - means Tehsil Officer Planning & Coordination, Tehsil Municipal Administration Chakwal.

xlv.	Verandah - means a roofed gallery, terrace or other portion of a building with at least one side open to courtyard or a permanent open space.

xlvi.	Zone - means an area/areas earmarked for particular use of character including an area in which sizes of plots have been prescribed.

PART -III

4.	Application of Building Bye-Laws.	 Every prospective builder who intends to carry out building works within the boundary of the area controlled by the Tehsil Municipal Administration shall comply with the requirements of these Bye-laws.

5.	Submission of Plans

i..	All applications shall be made on building application Forms A & B as 	prescribed by the Tehsil Municipal Administration at Appendix A & B.

ii.	The land shown as set-back on the building plan for street or road widening or parking shall vest in the Tehsil Municipal Administration Chakwal

iii.	No building application shall be entertained unless the plan is drawn by an Architect/Town Planner/Draftsman holding the license for the said purpose issued by the Tehsil Municipal Administration.

iv.	Only those Architects / Town Planners shall be registered who keep a Degree or Diploma in Architecture / Town Planning from a recognized University. Draftsman holding diploma as Civil Draftsman from a Government recognized Institute can be registered as Architect provided they keep work experience not less than three years and Tehsil Officer Planning & Coordination is satisfied with their practical performance.

v.	If at any time it is found that any licensed Architect / Town Planner / Draftsman has drawn any building plan against the site or is guilty of any such action which is prejudicial to the Municipal or public interest, the Tehsil Municipal Officer on the report of Tehsil Officer (Planning & Coordination) may cancel his license or may impose a fine on such Architect / Town Planner / Draftsman which may extend up to Rs.5,000/- (Rupees five thousands).

vi.	Every person intending to erect or re-erect a building shall give notice of 			such intention in writing to the Tehsil Municipal Administration by 			submitting Form - An appended to these Bye-laws, in order to 				obtain 	permission to execute work, shall at the same time submit:

a.	Certified / attested copy of the sale or gift deed, Farad, Jamabandi or other documents of title where under the applicant claims or derives ownership of the land or building sought to be erected or re-erected.

b.	Attested copy of the National Identity Card of the owner/owners of land.

c.	An affidavit declaring that builder will abide by building bye-laws of Tehsil Municipal Administration and that during the course of

Construction, he / she will not encroach upon any public or Government land.  That the documents provided by him / her along with building plan are correct and nothing has been concealed from the Tehsil Municipal Administration regarding ownership of the property / plot and that if at any stage it is proved that documents provided by him / her were incorrect, he / she will be solely responsible for this act with no liability accruing the Tehsil Municipal Administration.

d.	The specifications detailed in Form B appended to these bye-laws.

e.	A key plan drawn to a scale of 100 feet to an inch (1:1250) or other 			suitable scale to show the site to which it refers, showing all the 				roads and streets etc. leading to the proposed site from the nearest 				road or point of significance.

f.	A site plan on tracing cloth / paper, of the land on which it is intended to erect or re-erect the building, drawn to a scale of not more than 1/16th of an inch to a foot (1/16”: 1’-0”) and not less than one inch to 40 feet bearing an index explaining any special colours and symbols and shall be submitted in duplicate showing North position of the proposed or existing building.  The width and level of the streets or road on which the plot abuts, the adjoining plot numbers if any together with cardinal points and land use of the surrounding buildings should also be indicated.

g.	Plan, Section and Elevation of every floor including basement, if any intended to be erected which shall be drawn to a scale of not less than 1 inch to 8 feet (1:100) or if the building is so extensive as to make a smaller scale necessary not less than 1inch to 16 feet (1:200) .  Such Plans, Sections shall show the purpose for which the building and roof are intended to be used and access to and from several parts of the building and its appurtenances, the position from and dimensions of foundations, the proposed height of plinth and supper structure at the level of each floor, roof,

column, beam joints and girder, position of all proposed drains, privies, latrines, urinals and cesspools, position and dimensions of all projections beyond and main outer walls of the building, the scale on which the plan is drawn and the North sign.

(1)	All new works should be indicated on the plan by a Brown 	colour on the plan.

(2)	In both site plan and the building plan. all existing features, 	namely buildings and other land marks shall be shown  in 	BLUE, proposed new features Red and proposed 	demolition in Green.

(3)	For each floor plan a separate block plan will be shown on the drawing clearly dimensioned / indicating the covered area.

h.	Specifications and calculations of design regarding the material and method of construction to be used for external walls, partition walls, foundations, roofs, ceilings, floors, stair cases, fire places and chimneys.

i.	Every prospective builder who intends to make any addition or alteration to a building as a result of which the original covered area will increase or where changes are such as to alter the original plan, shall submit an application along with the documents giving detailed information about the alteration as required under these Bye-laws, and a certificate from a qualified structural Engineer to verify the stability of the building / structure, to the Tehsil Municipal Administration for permission to execute the work.

j.	Every construction plan submitted under these Bye-laws shall be prepared by a licensed Architects / Town Planners / Draftsman or under his supervision, shall bear his stamp, signature and registration number.

k.	Every Construction Plan for Commercial building under these bye-laws shall be prepared by the license Architects / Town Planners / Draftsman or under his supervision, and every prospective builders shall submit structural stability certificate of the buildings issued by structural Engineers shall bear his stamp, signature and registration number with Pakistan Engineering Council.

6.	Fees and Penalties

i.	Tehsil Municipal Administration shall charge fee for the scrutiny of building plans required to be submitted under these Bye-laws and other matters arising during the disposal of plans or in the course of its construction. This fee to be known as “Building Application Fee”  on covered area at the rate of Re.1/- per square foot,  or in such manner as may be revised from time to time in the

schedule. This fee shall be “non-refundable” whether the plan is approved or rejected.

ii.	Tehsil Municipal Administration may exempt from the payment of Building Application Fee, the premises which in the opinion of the Tehsil Municipal Administration will be used for religious or charitable purposes, but NOC from concerned authorities for the purpose shall be produced with other documents required for approval.

iii.	Serving and retired employees of the Tehsil Municipal Administration shall be exempted from the payment of Building Application Fee for their personal owned premises only once in life time.

iv.	If any developer of a building makes any violation from these bye-laws and approved plan, a penalty may be compounded on such violation by the Tehsil Officer Planning & Coordination as prescribed in the Annexure-A.

v.	The rate of Building Application Fee and Penalties can be revised from 	time to time by the Tehsil Municipal Administration.

7.	Registration of Application

Every building application plan and specifications referred to in the proceeding bye-laws shall be signed by the applicant and submitted to Tehsil Municipal Administration on  Form-A appended to these Bye-laws.  Applicant shall deposit advance building application fee, and on payment of such fees every building application fee shall be registered in the register of building applications maintained by Tehsil Municipal Administration along with Form-B appended to these bye-laws. If the applicant within 45 days of  the submission of plan, does not receive any notice of approval, disapproval,  amendment in the plan, provision of any other document etc. from the Tehsil Municipal Administration, shall send another notice to the Tehsil Municipal Administration for consideration of his application through registered mail.  And if within 15 days of the receipt of this registered notice, no action is taken/intimated by the Tehsil Municipal Administration, the plan shall  deemed to  be approved up to the extent  it does not contravene the provisions of these Bye-laws.

8.	Procedure, Notices, Inspection & Certificates During Construction.

- Every builder who carries out building works shall use sound building material which shall be of good quality and properly put together so as to ensure safety and stability of the building.

- A certificate of durability and in accordance with building plan be obtained from Architect and Engineers concerned after completion of each floor.

- Structure strength according to design be certified by the structural Engineer after on the spot inspection of the quality of the material especially of steel and concrete the completion of the building.

- In case of failure of the Structure Engineer should be held personal responsible.

v.	If on making inspection, the authorized officer/official of the Tehsil 	Municipal Administration finds that the building works:-

a)	Contravene any of the provisions of the Bye-laws, Tehsil Municipal Administration may by written notice require the person carrying out building work within a period to be specified in such notice, either to make such alteration as shall be specified in such notice, with the object of bringing the work in conformity with the said plans or provision of the bye-laws or get amended plans approved by Tehsil Municipal Administration after complying with the requirements of the bye-laws.

b)	In the event of non-compliance with the requisition as made under sub clause (i) above, Tehsil Municipal Administration may order cessation of work or order of demolition of such construction which may contravene any of the provisions of the building bye-laws and the expenses thereof shall be paid by the owner.

9.	a)       Completion Certificate

- Every prospective builder who carries out and completes building works as approved under these Bye-laws shall within 30 days of the completion of work apply in writing to Tehsil Municipal Administration together with the certificate signed by the licensed Architect/Town Planner/Draftsman, unless in the opinion of the competent authority such a certificate in not required.

ii.	Tehsil Municipal Administration shall issue a completion certificate on completion of building works provided the work has been carried out according to the sanctioned plans or deviation made therein during the construction are settled in advance in writing by an officer duly authorized in this behalf.

iii.	In case the prospective builder intends to construct the building in phases, the sequence of construction in phases duly numbered shall be indicated on the drawings.

b)	Lapse of Sanction

Every sanction for the erection or re-erection of building given or deemed to have been given by the Administration shall be valid for one year from the date on which it is given, and if the building so sanctioned is not begun within that period, it shall not thereafter begun unless the Administration on an application made therefore has allowed an extension of that period.

10.	The Making of Survey for the detection of buildings constructed in an unauthorized manner and taking of  action with regard thereto.

i..	Building Inspector shall make survey of the sites/buildings under construction from time to time with a view to:-

a.	To detect cases of contravention of the provisions of the Punjab Local Government Ordinance 2001, rules and these Bye-laws.

b.	To detect persons erecting or re-erecting buildings without permission.

c.	To ascertain how far the conditions of the sanction for erection or re-erection of building are being fulfilled.

And building inspector concerned will be responsible for above said duty under clause i(a),(b),(c).

ii.	All information’s regarding the correction, sanction or rejection of plans and inspection of site, any charges i.e. fee, fines and penalties etc. under these Bye-laws shall be communicated in writing to the persons concerned under the signature of Tehsil Officer Planning & Coordination.

- The result of every inspection under clause (i) above shall be recorded in the building form ‘C’ appended to these Bye-laws and brought to the notice of Tehsil Officer Planning & Coordination who may ensure that

adequate action has been taken with regard to each case of contravention of the Ordinance or Rule or Bye-laws framed there-under.

iv.	The authorized officer of Tehsil Municipal Administration may enter into or upon any premises with or without assistant in order to make any inspection, survey, or inquiry, provided :-

a.	no such entry shall be made between sunrise and sunset.

b.	no building used as a dwelling house shall be so entered unless with the consent of the occupants there of or after giving the said occupier at least six hours prior notice in writing of the intention to make such entry and of the purpose thereof.

c.	due regard shall always be had so far as may be compatible with the exigencies of the purpose for which the entry is made to the social and religious usages of the occupants of the premises entered.

d.	no claim shall lie against any person aforesaid for compensation for any damage necessarily caused by effecting such entry provided that force shall not be used for effecting any entry unless there are reasons to believe that an offense has been or is being committed against any of these Bye-laws.

PART - IV

11.	Building and Planning Control Requirements ( Residential).

i.	No portion of a building in a street or road shall be built to project beyond the existing line of frontage which in every case shall be taken to be outer wall of the building.

ii.	The building line of the proposed building abutting on any street or road in the existing localities shall be at a distance of half of the width of the said street or road from the centre of such street or road and no person shall erect or re-erect any part of the proposed building towards the street/road side abutting beyond the said building line.

iii.	In all the newly developed areas minimum street width will be 20 feet and every person intending to erect or re-erect a building shall have to provide minimum 10 feet wide space from his own land all along the proposed site for street/streets. In the areas in which a street/streets or road/roads already exist up to the proposed site or near thereby every person whose building or site abuts on the said street/streets or road/roads shall have to provide space equivalent to one half of the total width of the existing street/streets or road/roads from his own land.  In such cases the building line shall abut at a distance of 10 feet or half of the width of existing street/streets or road/roads from the centre of such street/streets or road/roads as the case may be.

iv.	Proportion of the site which may be occupied by the building and corresponding floor area ration shall be as specified in schedule - I.

v.	Three  feet hanging projection over the courtyard meant for passage shall not be included in the covered area.

vi.	The minimum number of storeys allowed to be constructed is four, excluding the basement, if constructed.

vii.	The top of the plinth of the building shall be at least one foot above the highest finished level of the adjoining road or street.

viii.	Every interior yard must be raised at least one foot above the level of the crest of nearest street or road so as to admit of easy drainage except floor of the porch, which should be at such a height from the street or road to allow easy entrance of the vehicle.

12.	Building Heights

i.	The height of any building for the purpose of these Bye-laws shall be measured from the top of the plinth to the top of the parapet wall.

ii.	No person shall construct any building abutting on any street:-

a.	of a height more than 24 feet if such street is up to 12 feet wide.

b.	of a height more than 44 feet if such street is 18 feet wide or more than 18 feet wide.

- the Administration / Nazim, may permit mumties, chimneys,

minarets of mosques, mobile phone tower, tower of churches, temples and similar structures to be constructed to a height in excess of the maximum height otherwise permissible under these Bye-laws.

iii.	Minimum space standard for building interiors shall be as per Schedule - II

COMMERICAL

13.	Height of the building

i.	Construction of more than 5 storey will not be allowed. Construction of basement is exclusive to this.

ii.	Maximum height of a commercial building shall not exceed 60 feet.

a)	Minimum effective height of any shop shall not be less than 9 feet 6 inches without any gallery (storage space) or 15 feet 6 inches with gallery (storage space).

b)	Minimum effective height of parking floors shall not be less than 8 feet.

c)	Minimum effective height of a parapet will shall be 2 feet 9 inches.

d)	all other floors shall have minimum effective height of 9 feet 6 inches. No open space on either side of the building shall be required.

iii.	Display of neon signs, advertisement etc. shall be on permissible places subject to the approval of the Administration. The Administration shall charge necessary fee on each sign, advertisement board at the rate and in a manner as fixed from time to time.

iv.	The minimum floor area of a shop shall be 100 sq.ft. having a minimum floor width of 8 feet.

14.	Set-back Requirements for Commercial & Semi-commercial Buildings

i.	In case of commercial and semi-commercial buildings every prospective builder shall be required to provide 4 feet wide set-back area on the front of the building situated on the following four major roads:

(1)	Rawalpindi Road.

(2)	Talagang Road..

(3)	Jhelum Road.

(4)	Bhaun Road / Sargodha Road.

In all other areas set back is exempted for commercial or semi-commercial buildings.

ii.	The land shown as set-back on building plans shall be surrendered in favour of Tehsil Municipal Administration through a surrender deed and it shall never be made a part of the building.

15.	GENERAL

i.	No land or building shall be used in a manner inconsistent with the use prescribed by the Administration in the approved plan.

ii.	No person shall start building works on a site abutting on a street / road without having first provided hoarding or barrier to the satisfaction of the competent authority along the same length of such site so as to prevent danger of injury to the public or the persons employed to the work however, this rule does not apply in the case of building work in connection with structures situated 15 feet away from edge of the public street and being not more than 25 feet high.

iii.	No part of any street shall be used in connection with construction repair or demolition of any building except with the written permission of the competent authority.  Any person holding such permission shall put up and maintain to the satisfaction of the concerned authority the fences or barriers in order to separate the building work from such street without interruption in the flow of traffic.

iv.	No construction material or debris shall be deposited in any street without the written permission of the competent authority on the condition that the owner will be responsible for clearing the street after completion of work. And if the owner keeps the building materials in any street without permission from TMA, he will be penalized @ Rs. 1.0 per Sq. ft per day.

v.	No excavation shall be made in any street without written permission of the competent authority on the condition that the owner will be responsible for clearing the street after completion of work.

vi.	No excavation shall be made in any street without written permission of the competent authority.  The applicant will inform all concerned departments/agencies about the date on which he intends to start excavation along with a copy of sanctioned plan/ the permission.

vii.	All materials, hoarding, fences or other obstructions in any street shall be kept clear of hydrants and other utility services, installations or alternative arrangements to the satisfaction of the competent authority and precautions shall be taken to divert or keep clear of obstructing any road side or drain during the period of temporary construction.

viii.	If any material, hoarding, excavation or any other thing in or near any street be in the opinion of the Tehsil Officer Planning & Coordination dangerous to the passengers along such street the competent authority shall cause the same to be removed, protected or enclosed so as to prevent danger there from and shall be entitled to recover the expenses thereof from the owner of such material or from the person who made such hoarding, excavation or the other thing to become dangerous.

ix.	The competent authority may give notice canceling any permission issued for the breach of any of the imposed conditions or for any other reason it may think fir and person holding the permission shall within 7 days of the

receipt of such notice have hoarding, fence or material removed and any excavation refilled or otherwise made good.

x.	All obstruction and erections in any street shall be removed with in 7 days of the completion of the work and the street and all drains and public utility installations shall be left in clean tidy and serviceable condition.

xi.	No excavation or earth work, demolition of a building which is likely to effect the stability of any adjacent building shall be started or continued unless adequate measures are taken before or during the work to prevent the collapse of any adjacent building or the fall of any part of it.

xii.	A site once excavated shall not be kept open beyond the period stipulated for completion of the work below ground level.

xiii.	Material shall not be placed or stacked near the edge of any excavation to endanger any person.

xiv.	No building may be demolished without a written permission from the competent authority.  No permit to demolish will be issued unless the Tehsil Municipal Administration is satisfied that the electricity, gas, water, sewerage or other utility services connections to the plot have been effectively cut off and such connections remain cut off during the period of the work.

xv.	No roof, floor or the parts of the building shall be so over loaded during the process of demolition / construction with debris of materials so as to render it unsafe.

xvi.	Every part of load shall be securely fixed or supported while being raised, lowered or suspended and shall be adequately secured to prevent danger from slipping or displacement.  Every receptacle used for raising, lowering and suspending blocks, bricks, tiles or other objects shall be so designed and constructed as to prevent accidental fall of such objects.

xvii.	In all buildings where the load of electricity would require installation of independent transformer / sub-station appropriate space for location and

access shall be provided within the premises as determined by the power supply agency.

xviii.	No tower / antenna be allowed to be erected on a building, whose basic building plan is not approved.

xix.	Any building plan approved by TMA, will not be applicable in any court.

16.	Inter Flooring

i.	Inter floor shall only be permitted in rooms other than those meant for habitation purposes, such as bath rooms, stores, kitchen pantries, passages, garges if combined with the main building, verandahs rooms meant for similar ancillary uses.

ii.	A minimum clear height of all the rooms referred to in clause a. above shall be as per schedule - II, except that the inter floor may have a clear height of 5 feet 6 inches when used as storage space.

iii.	Any horizontal division of a building so constructed as to be capable of used as living or sleeping room shall be considered to be storey for the purpose of these Bye-laws even though such division does not extend over the whole depth or  width of such building.

17.	Basement.

i.	Whenever a basement is constructed, it shall be subject to the fulfillment of the following conditions:

a.	That is shall be served with an independent entrance and shall have an emergency exit.

b.	That the level of the public sewer permits gravity flow at a gradient of not less than 1:40 or if this may not be possible pumping arrangements shall be installed.

c.	That minimum height of any basement room shall be 8 feet.

d.	That the drainage passing under the basement is gas tight.

e.	The minimum area of the basement shall not be less than 100 sq. 	ft.

f.	The foundation of the basement shall not intrude into adjoining property.

18.	Internal Lighting & Ventilation Including Mechanical

Ventilation

i.	Every room other than rooms used predominantly for the storage of goods shall, except where mechanical arrangements are provided, have a combined glazed area of not less than 12% of the floor area of such room and 50% of such opening shall be capable of allowing free uninterrupted passage of air.

ii.	Every latrine, water closet, urinal stall and bath room shall be provided with natural lighting and ventilation by means of one or more openings in external walls having a combined area of not less than 2 sq. ft. per water closet, urinal stall or a bath room and such openings shall be capable of allowing free uninterrupted passage of air.

iii.	Kitchens, lavatories, WCs and bath rooms may have sources of daylight and natural ventilation from internal air wells of adequate size.

iv.	Where permanent mechanical air conditioning is intended to be provided, the relevant building Bye-laws dealing with internal lighting of rooms maybe waived off at the discretion of the competent authority.

19.	Meaning  of Escape in case of Emergency

i.	In every multi-storied building, emergency stair case shall be provided in addition to the main stair case/stair cases.

ii.	All means of escape from a building including extra corridors stairs etc. shall permit unobstructed access to the street.

iii.	An emergency stair case shall be sited at such a position that it should be accessible  to all the unit without any hindrance or obstruction and it should be open to a permanently ventilated space.

iv.	All buildings shall have windows on the street elevation within the convenient reach and of adequate size to enable persons to escape in case of emergency.

20.	Building Structures

i.	Sites

a.	No building shall be erected upon a site reclaimed with town sweeping or other refuse until the whole ground surface or site of such building has been rendered or become innocuous and has been covered with a layer of clean earth, sand, hard core clinker or ash rammed solid at least 12” thick.

b.	Boundary walls which abut on public streets, footways or places which the public are allowed to use shall not have fencing consisting of barbed wire or any material likely to cause injury to humans or animals.

ii.	Foundations

a.	The foundation of any building shall be wide enough to allow the distribution of load on each sq. ft. not exceeding one and a quarter ton.  No building shall be built upon newly filled earth till it had ample time to settle down and not even then without the special permission of the Tehsil Municipal Administration, Chakwal.

b.	The owner shall cause tests to be made to prove the nature of the soil, wherever considered necessary by the competent authority.  Such test must be made for all sites intended to be constructed upon with buildings having more than 3 storeys.

c.	Where a building is to be erected near a drain or an excavation at a distance of less than the depth of the said drain or excavation the owner shall satisfy the administration that the foundations of the building are safe.

d.	The owner shall submit structural calculations or a certificate from a qualified structure engineer to verify the foundations and super structure, if required by the competent authority.

e.	Proper damp-proofing shall be provided for walls and floors according to the standard specifications.

f.	Where the floor or wall of a building is, in the opinion of the competent authority, subject to water pressure that portion of the building below ground level shall be suitably water-proofed.

iii.	Stair Cases and Lifts

a.	All residential buildings shall have stair cases having a minimum clear width of 3 feet.

b.	In commercial buildings stair cases shall have the following minimum width for all storeys:

Upto 3 storeys		-	3 feet and 6 inches

Above 3 storeys	-	4 feet

c.	The riser of stair case steps shall not be more than 8 inches and the tread not less than 10 inches.

d.	A landing shall not be less than 3 feet in depth.

e.	All stair cases in multi-storied buildings shall be of re-inforced cement concrete or other non-inflammable material.

f.	Lift shall be essential if the building has more than 3 floors above the ground floor.

21.	Fire Resistance and Fire Precautions

i.	A building or any structural part of a building shall be deemed to have an adequate standard of fire resistance.

ii.	Every garage shall be constructed of fire resisting materials of good standard.

22.	Miscellaneous

i.	No street shall be closed which is existing at site.

ii.	Sanction shall be required from Tehsil Municipal Administration Chakwal for the construction of boundary wall around the open plot after paying prescribed fee.

iii.	In all the cases where the building is already in existence (old construction) regulation charges shall be equivalent to prevalent building fee for the purpose of sanction by Tehsil Municipal Administration Chakwal if it does not violate the space standards formulated under these bye-laws.

- PT-I Form / PTD Form as title documents shall be acceptable for the city area i.e. within circular road incurring old Abadi, if any subject to the verification by the Naib Union Nazim of the said area.  Moreover, it shall be advertised in the newspaper for information or objections of the general public.

- Building plan shall be admitted only if it is in accordance with the prevailing building Bye-laws and subject to the verification of site by the building inspector concerned.

vi.	NAZIM, Tehsil Municipal Administration Chakwal may relax any provision of these Bye-laws, but after recording solid reasons.

vii.	These Bye-laws shall extend to the whole of the Tehsil Chakwal, but kacha buildings shall be exempted subject to the condition that no encroachment is being made upon any public / private land.

viii.	Any policy / direction pertaining to these Bye-laws, issued from time to time, by the Government shall be considered part of these Bye-laws.

APPENDICES

- SCHEDULE - I	-

- SCHEDULE - II	-

- ANNEXURE - A	-

Building Application Fee and Schedule

Composition Charges

- APPENDIX - A	-

Form - A

- APPENDIX - B

-  Form - B

- APPENDIX – C

-  Form - C

- APPENDIX - D

- Reports

SCHEDULE – I

Plot Size			          MAX. BUILT UP AREA ON

AREA IN MARLAS		      GROUND FLOOR (RESIDENTIAL)

Upto 3 marlas 	No restriction if proper ventilation is provided.

Above 3 marlas upto 10 marlas			80 %

Above 10 marlas upto 20 marlas			70 %

Above 20 marlas					65 %

Apartment Buildings					65 %

SCHEDULE - II

MINIMUM SPACE STANDARDS FOR BUILDING INTERIORS

Space  		Min. Area		Min. Height		Min. Width

Habitable Room

i.e. Bed Room,

Drawing Room,	   100 sq.ft.		    9’ - 0”		    8’ - 6”

Lounge,

Servant Room etc.

Kitchen		    45 sq.ft.		    8’ - 6”		    5’ - 0”

Baths & Toilet		    24 sq.ft.		    7’ - 0”		    3’ - 0”

Garages & Porches	          -		               7’ - 6”		    9’ - 0”

Basements : If used for habitable purposes the rules for respective space use will be applied.

Note 	     : Sq. Ft. means square foot.

ANNEXTURE - A

BUILDING APPLICATION FEE

Every prospective builder has to submit fee along with application as prescribed below:

S.No.		Category					Fee on Covered Area

1.		Residential						Rs. 1.00 / Sft.

2.		Commercial						Rs. 3.00 / Sft.

3.		Industrial						Rs. 2.00 / Sft.

4.		Housing Scheme					Rs. 3000 / Acre

5.		Construction of Boundary Wall			Rs. 25.00 / Marla

6.		Registration & Renewal Fee of Architects/		Rs. 2000/-

Town Planners / Draftsmen/Structural Engineer

License Fee who are not registered with PCATP

and PEC.

COMPOSITION CHARGES

S.No.		Nature of Deviation				Composition Charges

1.	Un-authorized and illegal construction that form an		Rs. 10 / Sft.

integral part of the authorized building but is in

excess of permissible covered area.

2.	Un-authorized and illegal construction in excess of		Rs.   5 / CfT.

the prescribed height limits (area will be measured

in cubic feet).

3.	Construction in any form including excavation for		Equal to actual fee.

construction prior to approval of plan.

4.	Un-authorized change in the character of use of a		Rs.   5 /  Sft.

building or part there of from residential use to

commercial or industrial or office use.

Notes :  i.	In case where the owner does not apply for compounding the offense committed by him or fails to pay penalty at the prescribed rates plus composition fee, it shall be open to the Administration or any person authorized by it on his behalf to demolish the offending structure.

ii.	In case an un-authorized construction is under way, it shall be open to the Administration or any person authorized by it in this behalf to decide whether a daily fine upto Rs.1500/- may be imposed in addition to any other penalty.

APPENDIX - A

FORM - B

DETAIL OF THE PROPOSED BUILDINGS

1.	Erection or re-erection of the building______________________________________________

2.	Plot/Building No. which is sought to be erected/re-erected______________________________

3.	Use of the proposed building_____________________________________________________

4.	No. of storeys_________________________________________________________________

5.	Approximate No. of persons who are likely to inhabit the building________________________

6.	No. of latrines/bath rooms	_______________________________________________________

7.	If any building or portion of the building already exists on the site. Whether its plan

is being submitted ?____________________________________________________________

8.	Source of water supply for the purpose of construction_________________________________

9.	Is the entire construction on the applicant’s	own land ?________________________________

Signature of the applicant     ________________________						Name of the applicant    ___________________________

Father/Husband’s Name  ___________________________

Address ________________________________________

______________________________________________

APPLICATION FOR ERECTION / RE-ERECTION OF BUILDINGS

Name of the applicant ____________________________________________________

with parentage and _______________________________________________________

address _______________________________________________________________

Total Area of the plot	 ______________________________________________

Area of the Basement Floor under construction ____________________________

Area of the Ground Floor under construction ____________________________

Area of the First Floor under construction      ____________________________

Area of the Second Floor under construction ____________________________

Area of the Third Floor under construction    ____________________________

Area of the Fourth Floor under construction    ____________________________

Area of the Fifth Floor under construction    ____________________________

Area of the Mumty under construction         ____________________________

Total Covered Area		=	           ____________________________

Building Fee					Rs. _______________________

Total Amount		Rs. ________________________

Building Inspector

Admit the plan in the relevant building register after collecting the above charges.

Received Rs. _______________Vide Receipt No. ______________ Dated __________

Building Clerck

Appendix -C

FORM - C

1.	Serial No. of  building application		______________________________

2.	Date of receipt of application			______________________________

3.	Date of expiry of limitation			______________________________

4.	Name of applicant, parentage and		______________________________

full address					______________________________

______________________________

5.	Situation of building				______________________________

6.	Particulars of building				______________________________

Building Clerk

Draftsman

Planning and Coordination Branch,

Tehsil Municipal Administration,

Chakwal

Appendix D

REPORTS

1.	The dimensions shown on the plan		______________________________

are in accordance with the site.

2.	The street width shown in the plan is		______________________________

in accordance with the site.

3.	Are all the building lines according to the	______________________________

prescribed by the building bye-laws ?

4.	Is there any  objection from sanitation	 ____________________________

point of view ?

5.	Does the proposed building conflict with	______________________________

any town planning scheme of any

Government Department ?

6.	Is the plot/property area shown in the 	 _____________________________

drawing in accordance with attached documents ?

7.	Is there any encroachment in the existing	______________________________

building ?

8.	Is the Key plan/Location plan according	______________________________

to the site ?

9.	Is the applicant owner of the existing		______________________________

building/proposed site ?

10.	Does the proposed or existing building 	______________________________

involve any encroachment on Municipal

land, street, road, drain, graveyard, or land	______________________________

owned by the Government or any other local

authority or person ?

11.	What is Construction Position at site?	______________________________

Building Inspector / Draftsman

11.	Recommendations / observations	____________________________________

with regards to the scrutiny of the

building plan				____________________________________

____________________________________

____________________________________

Tehsil Officer (Planning & Coordination)

12.	Recommendations / observations	____________________________________

____________________________________

____________________________________

Tehsil Municipal Officer

13.	Final Orders with signature and date	____________________________________

____________________________________

____________________________________

Tehsil Nazim Chakwal

FORM - Z

File No.__________

From	 ___________________________________________

___________________________________________

Page No.

N I Card _________________________________________	       1

Form  ‘A’ ________________________________________	       2

Application for Erection or Re-erection of Building ________	       3

Form ‘B’ _________________________________________	       4

Form ‘C’ _________________________________________	       5

Reports ___________________________________________	       6 & 7

Affidavit __________________________________________	       8

Ownership Deed / Title ______________________________	       9

Original Map (prepared on tracing cloth / paper) ___________	      10

Two Duplicate Copies of Map _________________________	      11 & 12

Any other Document: ________________________________

__________________________________________________

Appendix - A

FORM - A

From  	____________________________________

____________________________________

To,	The Tehsil Officer Planning and Coordination,

Tehsil Municipal Administration,

Chakwal.

Through this application under Section 24 of the Punjab Local Government Ordinance 2001, I hereby inform that I intend to erect/re-erect the building on this site bearing Khasra No.______________ situated in Mohallah ______________________________ Mouzia ___________________ as per details mentioned in the Form B attached herewith. I undertake not to start the erection/re-erection of the building before grant of sanction by the Tehsil Municipal Administration Chakwal.  Following documents are also annexed:

a)	Complete building plan on tracing cloth/paper in which all the requisite details of the building have been given in accordance with the building bye-laws. The plan has been drawn by the licensed architect of Tehsil Municipal Administration, Chakwal.

b)	Original/Certified copy of the title deed and Fard Jamabandi under which I claim title to the site sought to be erected/re-erected (give the description of the title deed).

c)	I undertake that keeping in view the level of the street/drain existing nearby I shall construct the drain and pave the street in accordance with the plan at my own cost. I shall use the water from well and shall not claim the water supply from the Tehsil Municipal Administration, Chakwal till such time that water supply is arranged by the Tehsil Municipal Administration for this locality as a whole.

d)	I shall not claim any other municipal / civic service, until and unless it has been extended to the whole locality.

Signature of the applicant __________________________

Name of the applicant _____________________________

Father’s/Husband’s Name __________________________

Address ________________________________________

_________________________________________